|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/SOP/01 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Metallizer Process Flow Chart | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Define the complete metallization process flow from bare film receipt to finished jumbo dispatch, ensuring all quality checkpoints are covered.

📋 Process Flow — Step by Step

| Step | Process Activity | Responsible |
| --- | --- | --- |
| 1 | Receive bare film from Primary Slitter area | Operator |
| 2 | Verify roll ID against production plan (Format MET/F/04) | Operator |
| 3 | Select shield plate per roll width, apply graphite suspension paint | Operator |
| 4 | Load roll on unwinder, thread film through web path | Operator |
| 5 | Close chamber, start pumpdown sequence | Operator |
| 6 | Boat warmup at target current/voltage, verify via viewing window | Operator |
| 7 | Start production run — set OD target, enable AUTO mode on sensor | Operator + Supervisor |
| 8 | Monitor OD (L/C/R), speed, tension, vacuum during run | Operator |
| 9 | Take periodic samples for offline QC check (OD, pinholes, scratch) | QC Officer |
| 10 | End of roll — stop evaporation, vent chamber, unload jumbo | Operator |
| 11 | Record all parameters in logbook (Format MET/F/01) | Operator |
| 12 | Enter production data in SAP (T-Code: ZPP_METJUMBO) | Supervisor |
| 13 | Transfer jumbo to slitting area via conveyor | Operator |

⚠️ Every step has a quality gate. No jumbo proceeds to slitting without supervisor sign-off on logbook entries.