|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/SOP/02 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Base Film Roll Movement at Shop Floor | Scope: Metalliser Area |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe and damage-free movement of base film rolls from Primary Slitter area to Metalliser loading zone.

👤 Responsibility

Metallizer Operator and Trainee Operator.

📋 Procedure

Step 1 — Conveyor Transfer

Take roll from Primary Slitter using conveyor system (Conveyor Nos. as per plant layout). Operator must visually confirm roll ID matches planning format before lifting.

Step 2 — Overhead Crane Lifting

Lift the roll with core plugs at both ends using overhead crane with sling belt capacity more than 5 MT.

⚠️ Ensure to wear Safety Helmet during ALL crane operations. No exceptions.

Step 3 — Sling Belt Inspection

Check sling belt for damage before every lift. Damaged sling belt = serious accident risk. Replace immediately if any cut, fray, or deformation found.

Step 4 — Roll Protection

While lifting rolls, use foam sheets at both ends so that the roll surface is not damaged by sling belt pressure. Place roll on designated pallet — never directly on floor.

⚠️ Never leave a roll suspended on crane unattended. Complete the movement in one continuous operation.