|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/SOP/08 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Evaporation Source — Copper Clamp Replacement | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🔧 Required Spares & Tools

1. Copper Clamp (new)

2. Hex Bolt and Torque Wrench / Spanner (for tightening clamp bolts)

3. Scotch-Brite Pad (for cleaning bus-bar / clamp surface)

📋 Procedure — Steps to Follow

Step 1 — Confirm Pneumatic Valve Closed

Go to SCADA screen. Wait until Pneumatic Valve indicator turns RED. Red = valve fully closed. Do NOT proceed until valve is confirmed closed.

Step 2 — Close Main Butterfly Valve

Locate the main butterfly valve. Close it by hand completely to stop water flow to the evaporation source section.

Step 3 — Drain Remaining Water

Open the small butterfly valve (drain valve). Allow all remaining water in the pipe to drain completely.

Step 4 — Remove Damaged Clamp

After water has fully drained, carefully remove the burnt or damaged copper clamp. Do NOT touch hot surfaces or wet areas with bare hands.

Step 5 — Clean Contact Surfaces

Use Scotch-Brite Pad to thoroughly clean the bus-bar and clamp mating surface. Remove all black carbon deposits, dust, and degraded coating. Clean surface = good electrical contact = uniform boat heating.

Step 6 — Install New Clamp

Fit the new copper clamp correctly. Tighten hex bolts with torque wrench evenly. Uneven tightening causes hot spots and premature boat failure.

Step 7 — Restore Valve Positions

Return all valves to their original running positions: Open the water supply valve to source. Close the drain valve. Verify no leaks at clamp connections.

⚠️ Machine can be started ONLY after all valves are correctly set and verified. Incorrect valve position = flooding or dry running = major equipment damage.