|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/SOP/10 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Aluminium Dust Waste Disposal | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe collection, handling, and disposal of aluminium dust waste generated from metalliser source sections during setup and changeover operations.

📋 Procedure

Step 1 — Collection During Setup

Aluminium dust is scraped out of the Met-1 and Met-2 source sections using dedicated scraping tools during setup time.

Step 2 — Primary Collection

Dust is collected in metal containers of 20 litre capacity at the machine site during setup work.

Step 3 — Transfer to Storage Container

After setup is completed, transfer dust from the 20-litre container to the larger 500-litre storage container placed in the designated area.

Step 4 — Disposal to Scrap Yard

When the jumbo bag inside the 500-litre container is full, it is shifted to the scrap yard using a forklift.

🛡️ Critical Safety Rules

⚠️ NEVER move aluminium dust while it is raining outside.

⚠️ NEVER store aluminium dust in open areas where it could encounter water.

⚠️ Aluminium dust + water = violent exothermic reaction = EXPLOSION risk. Use ONLY Class D dry powder extinguisher or dry sand in case of fire. NEVER use water or CO₂.