|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/03 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Safety — People Working Along with Moving Parts | Scope: Metalliser Department |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Ensure safety of all personnel working near moving parts of the metallizer, conveyor systems, and winding mechanisms.

🛡️ Critical Safety Rules

1. Never place hands near any rotating roller, drum, or shaft while the machine is running.

2. Always tie back loose clothing, hair, and remove jewellery before approaching the machine.

3. Conveyor system: Keep hands on the control button at all times during operation. Do not release until conveyor has completely stopped.

4. Winding cart area: Never stand between the winding cart and the chamber during cart movement.

5. Chuck/shaft area: Never reach into the chuck zone when air pressure is engaged. Sudden chuck release can crush fingers.

6. Use LOTO (Lock Out Tag Out) before any maintenance work inside the chamber or near rotating parts.

7. Emergency stop buttons: Know the location of ALL emergency stops on the machine. Test monthly.

⚠️ PPE mandatory at ALL times: Safety helmet, steel-toe shoes, gloves, safety goggles. No PPE = No entry to machine area. Zero exceptions.