|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/05 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Fixing the Shaft in Bare Roll for Metallization | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safely install the winding shaft into bare film rolls before loading onto the metallizer unwinder.

👤 Responsibility

Metallizer Operator.

📋 Procedure

1. Place the bare roll on the roll cradle/support stand. Ensure the roll is stable and will not move.

2. Select the correct shaft diameter matching the roll core ID (6 inch or 8 inch as per machine).

3. Insert the shaft through the core carefully. Do not force — if the shaft does not slide through smoothly, check for core damage or wrong shaft size.

4. Engage the shaft locking mechanism (air grip or mechanical lock). Verify the roll is firmly held and does not slip on the shaft.

5. Check that the shaft extends equally on both sides of the roll for balanced lifting.

6. Attach core plugs at both ends to protect the shaft during crane transport.

⚠️ Never leave a roll on the shaft without locking. An unlocked roll can slide off during crane lift = serious accident risk.