|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/06 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Loading & Unloading Roll in the Machine | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe procedure for loading bare film rolls onto the unwinder and unloading metallized jumbos from the rewinder.

📋 Unloading Metallized Roll (Rewinder Side)

1. Set the tension to OFF on the rewinder.

2. Cut the film to free the metallized roll from the web path.

3. Unlock the chucks from both sides of the rewinder shaft.

4. Bring the overhead crane and fix the hanger on the metallized roll shaft.

5. Ensure both side hangers are fixed properly BEFORE lifting. Double-check attachment.

6. Take the rewind roll out of the machine VERY SLOWLY. Place on the conveyor belt.

7. Ensure no metal pieces, oil, or grease on the belt that could contaminate the metallized roll.

📋 Loading New Roll (Unwinder Side)

8. Shift the empty unwinder core shaft to the rewinder side and chuck properly for the next metallized roll rewinding (using crane).

9. Place the new bare roll on the unwinder chuck and lock properly.

10. Thread the film through the web path as per the required metallization side.

⚠️ Never rush the unloading process. A dropped jumbo = damaged roll surface + potential injury. Slow and steady saves the roll and the operator.