|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/07 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Release of Aluminium Stray Dust, Heat & Noise | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safety procedures for handling aluminium dust exposure, heat hazards, and noise protection during metallizer operations.

🧹 Aluminium Dust Safety

1. Exhaust fan MUST be operated ON when the winding cart moves out of the chamber.

2. Do NOT enter the chamber immediately after opening. Wait minimum 1 minute until hot vapors are sucked out by the exhaust fan.

3. Wear ALL protective equipment: respiratory mask, gloves, and safety goggles while working inside the vacuum chamber and winding section.

4. Take pauses while cleaning the diffusion pump internal area. If you feel congestion, immediately come out to fresh air.

5. Collect all stray aluminium scrap and put it in the designated bag. Ensure it does NOT encounter water.

6. If aluminium scrap accidentally meets water, it must be completely dried before packing.

7. Transfer aluminium dust and stray aluminium to stores for disposal on weekly basis.

8. Do NOT touch any hot machine parts without wearing heat-resistant hand gloves.

9. Cleaning agent (acetone) must be kept away from the machine and stored in closed containers.

🔇 Noise Protection

1. Always use ear plugs while the machine is in running condition.

2. In case of any abnormal sound from the machinery, immediately inform the maintenance department.

⚠️ Aluminium dust + water = violent exothermic reaction = EXPLOSION. Class D dry powder extinguisher or dry sand ONLY. NEVER use water or CO₂ on aluminium fires.