|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/08 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Releasing & Storage of Used Paper Tape | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe removal, handling, and storage of used paper masking tape from the chill drum during boat changeover.

👤 Responsibility

Metallizer Supervisor and Metallizer Operator.

📋 Procedure

1. Ensure shields are set properly so that paper masking tape is not exposed to excessive heat during the run.

2. Remove the paper tape gently from the drum. Avoid abrasion while collecting it — rough handling creates particles.

3. While cleaning the chill drum with acetone, ensure NO used paper tape is left on the drum surface.

4. Put the peeled-off paper tape in the metallic dust bin and close it properly.

5. Wear safety goggles while removing paper tape to avoid eye injury from tape fragments.

6. Ensure used paper tape does NOT encounter acetone — fire/flash hazard.

7. Use hand gloves at all times. Keep acetone-soaked gloves AWAY from used paper tape.

8. Wear respiratory mask during the cleaning process.

⚠️ Used paper tape + acetone = fire hazard. Keep them completely separated at all times.