|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/13 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Shift In-charge Responsibilities — Metalliser | Scope: Metalliser Department |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Define the key responsibilities of the Metalliser Shift In-charge to ensure production targets, quality standards, and safety compliance are maintained every shift.

📋 Core Responsibilities

1. Machine & SCADA Monitoring

Visit both machines and verify SCADA entries: OD, Width, Micron, Spreader Angle, Tension values. Ensure operator is entering correct Roll Data.

2. Production Target

Shift production target = 10 Rolls. Mix: 11 Rolls (OD 2.2) + 10 Rolls (High OD) per day across shifts. One boat cycle target = 8 Rolls (4 HOD + 4 NOD) or 10 Rolls (all NOD).

3. Planning Execution

Ensure both Metalliser and Slitter production follows the written planning schedule exactly.

4. Downgradation Control

Target = 0% Downgradation. If any roll is downgraded, complete root cause analysis and corrective action is mandatory.

5. Raw Material Control

Check availability and consumption of evaporation boats, aluminium wire, and O₂ cylinders.

6. Quality Check with QC

Inspect for Low OD, Pinholes, Crease, and Scratches (plain side) together with QC team under proper inspection lighting.

7. Tools & Conveyor Management

Verify tools are available as per list. Keep all conveyors empty and clean.

8. Logbook & Cleanliness

Ensure all logbooks are fully filled and signed. Share cleanliness photos on the department group.

⚠️ Shift In-charge is the single point of accountability for everything that happens on the floor during their shift. No delegation of responsibility.