|  | VKS TECH Met Operational Excellence | Doc: VKS-MET-WI-24 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Metallised Waste Reduction & Control | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Waste Target

Total Metallised Waste Target: 1% or below (mandatory).

👷 Metalliser Operator Responsibilities

Roll Start-Stop Waste Control

Start Waste = 90 m, Stop Waste = 10 m, Total = 100 Metres

% Loss = (100 x 100 / 72000) = 0.138%

⚠️ Above this = Operator Responsibility. Record reason in logbook.

Trim Waste Control

Total Trim = 15 mm maximum (both sides combined).

% Loss = (15 x 100 / 2900) = 0.535%

Web alignment must be perfect — maintain edge-to-edge OD uniformity.

B-Grade Waste = 0% (Zero Tolerance)

No Low OD, No Pinholes, No Scratches, No Crease, No OD Bands.

✂️ Slitter Operator Responsibilities

Sample Cut Discipline: 3 Samples per Jumbo = 60 m ONLY (7 Layers, 20 Metres/Set).

Loss = (60 x 100 / 72000) = 0.083%

Extra sample cutting is strictly prohibited.

📊 Mandatory Targets — Summary

| Waste Category | Limit | % Loss |
| --- | --- | --- |
| Start + Stop Waste | 100 Metres | 0.138% |
| Trim Waste | 15 mm / 2900 mm | 0.54% |
| Slitter Sampling Waste | 60 Metres (7 layers) | 0.083% |
| B-Grade Waste | ZERO | 0% |
| TOTAL TARGET | MANDATORY | ≤ 0.8% |

⚠️ Every day waste and reason must be recorded in logbook. Target 0.8% is mandatory — not aspirational.