|  | VKS TECH Met Operational Excellence | Doc: VKS/MET/WI/17 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Met Jumbo Production Entry in SAP | Scope: Met-1 & Met-2 |
|  | vkstech.com ¦ Free Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Step-by-step procedure for entering metallized jumbo production data into SAP system for inventory tracking, billing, and material consumption recording.

👤 Responsibility

Operator: Maintain logbook with correct timings and operating parameters.

Supervisor: Enter production data in SAP using logbook. Correct any logbook anomalies before entry.

📋 Procedure — Step by Step

Step 1 — Open SAP & Login

Open SAP software. Login with assigned Production Entry credentials.

Step 2 — Create Production Order (T-Code: COR1)

Fill Material Type (starts with 3SF-), Plant code, and Quantity (e.g., 8500 Kg for 2 rolls). Select Process Order type: Met-1 or Met-2 as applicable. Press Enter and Release the order.

Step 3 — Open Met Jumbo Entry (T-Code: ZPP_METJUMBO)

Enter the Production Order number created in Step 2. Enter Plant code. Execute.

Step 4 — Fill Production Details

From logbook, enter: Posting Date, Base Roll ID, Consumed Length, Met Width. Enter all timing details: Chamber Closing time, Vacuum time. Verify Machine Speed is correct. Enter Supervisor names and Downtime (in minutes) with reason codes: P=Process, M=Mechanical, E=E&I, O=Others.

Step 5 — Base Film Consumption

Click Base Film option. Enter the weight of bare jumbo consumed during metallization.

Step 6 — Process Jumbo

Click "Process Jumbo" button to generate the metallized jumbo record.

Step 7 — Raw & Waste Entry

Click "Raw & Waste" option. Use the calculation Excel sheet to compute wire consumption, waste percentages, and material balance. Enter all values accurately.

Step 8 — Save & Confirm

Press Ctrl+S to save. Confirm all entries. Production order is now complete in SAP.

⚠️ Enter data in the SAME shift — do not delay. Grade correctly (2=OK, 3=B-grade, 5=Salvage, 6=Scrap). Any discrepancy → inform shift supervisor immediately. Accurate SAP entry is CRITICAL for inventory and billing.