|  | VKS TECH Met Operational Excellence | Doc: VKS/MET_SL/WI/01 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Roll Selection and Load on Unwinder | Scope: Met Slitter-1 & 2 |
|  | vkstech.com ¦ Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe selection, transport, and loading of metallized jumbos onto the secondary slitter unwinder per the slitting plan.

👤 Responsibility

Slitter Operator (primary). Slitter Supervisor (verification).

📋 Procedure

Step 1 - Check the Slitting Plan

Open daily slitting planning sheet. Identify which jumbo is scheduled. Confirm customer, material code, width requirements, core specifications.

Step 2 - Verify Treatment Side

Check planning for metallization side (Inside or Outside). This determines roll orientation on unwinder. Wrong orientation = all slit rolls have metal on wrong side = full rejection.

Step 3 - Read Roll Label and Job Card

Before touching the roll, read the physical Job Card. Look for: QC remarks, defect notes from metallizer operator, OD readings, downgrade flags. If Job Card mentions ANY defect, inform supervisor BEFORE loading.

Step 4 - Position Roll Behind Machine

Use overhead crane to bring jumbo behind slitting machine. Safe crane operation: sling belt checked, foam at both ends, helmet worn, clear path confirmed.

Step 5 - Fix Unwinder Chuck

Using back panel control or remote, engage unwinder chuck into roll core. Verify firm grip - chuck must be fully locked on both sides.

Step 6 - Raise Roll onto Unwinder

Operate unwinder lift mechanism. Verify alignment - roll must be centered, not skewed.

Step 7 - Thread Film Through Web Path

Pass film through entire web path using drive chain threading system. Ensure film passes over all guide rollers and tension rollers correctly.

NEVER skip reading the Job Card. If it says "Low OD edge 20mm" or "Pinhole cluster at 3000m" - that changes how you slit this jumbo. Ignoring it means producing B-Grade rolls that could have been saved.