|  | VKS TECH Met Operational Excellence | Doc: VKS/MET_SL/WI/02 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Slitting Machine Setup | Scope: Met Slitter-1 & 2 |
|  | vkstech.com ¦ Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Complete and accurate machine setup before starting any slitting run, ensuring correct widths, blade alignment, tension settings, and quality standards.

👤 Responsibility

Slitter Supervisor (setup verification). Slitter Operator (execution).

📋 Machine Setup Steps

Step 1 - Feed Slit Parameters

Enter all parameters in machine HMI: slit widths, core width, core ID (3/6/8 inch), arm selection, nip roll selection, blade type, tension values, pressure values for each zone.

Step 2 - Select Arm and Nip Roll

Position slitting arms at correct width intervals. Select and engage appropriate nip rolls for each slit position.

Step 3 - Adjust Blade

Set blade depth and angle for the film type and thickness. Too deep = cuts into core. Too shallow = incomplete separation.

Step 4 - Fix Cores

Place paper cores of correct ID and wall thickness into each arm. Cores must sit flush and centered - misaligned core causes telescoping.

Step 5 - Inch Film and Align

Inch film forward slowly. Verify each blade cuts at exact planned position. Adjust micro-positioning if any blade is off.

Step 6 - Verify Blade Sharpness

Check each blade produces clean cut with no rough edges, tearing, or partial separation.

Step 7 - CROSS-CHECK Widths

Before pressing Start, physically measure EVERY slit width using measuring tape. Compare each against planning sheet. This is the final quality gate.

✅ Quality During Run

1. Monitor for rough cutting, wrinkles, ripples, edge curl, surface defects. 2. After slitting, ALL rolls checked by QC officer. 3. Wrap: metallized film (metal-out) + Blue Air Bubble 100 GSM. 4. Weigh every roll. 5. Enter SAP data.

Manual width verification with measuring tape is MANDATORY before every run. Never trust only the digital HMI readout - machines drift. One wrong width = entire batch rejected.