|  | VKS TECH Met Operational Excellence | Doc: VKS/MET_SL/WI/03 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Blade Change Procedure | Scope: Met Slitter-1 & 2 |
|  | vkstech.com ¦ Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe and correct blade replacement to ensure clean, defect-free cutting. Dull or chipped blades create edge defects on every roll until replaced.

👤 Responsibility

Slitter Operator (execution). Supervisor (verification and logbook sign-off).

📋 Procedure

Step 1 - Stop Machine and LOTO

Bring machine to complete stop. Engage Lock Out Tag Out. Never change blades while machine can be accidentally started.

Step 2 - Wear Cut-Resistant Gloves

Slitting blades are extremely sharp. Cut-resistant gloves are MANDATORY. Regular cotton gloves will NOT prevent a blade cut.

Step 3 - Loosen Blade Holder

Using designated tool, loosen blade holder clamp carefully. Support blade so it does not fall.

Step 4 - Remove Old Blade

Remove old blade and place directly into used blade container. Never place used blades on machine surface, floor, or pocket.

Step 5 - Inspect New Blade

Before installing, visually inspect for chips, cracks, or edge damage. A damaged new blade is worse than a dull old one. Reject if any defect found.

Step 6 - Install Correctly

Place new blade in holder with correct orientation (cutting edge direction) and correct seating depth. Too deep = cuts core.

Step 7 - Tighten Evenly

Tighten blade holder clamp evenly. Uneven tightening causes blade tilt = angled cuts = damaged roll edges.

Step 8 - Verify with Inch Test

Remove LOTO. Inch film slowly. Inspect cut: clean separation, no rough edges, no tearing, no partial cuts.

Step 9 - Record in Blade Logbook

Record: Date, old blade serial, new blade serial, machine position, operator name. Supervisor signs off.

Check blade sharpness after every 5 jumbos minimum. A dull blade silently degrades edge quality until customer rejects the batch.