|  | VKS TECH Met Operational Excellence | Doc: VKS/MET_SL/WI/05 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Joint Formation at Secondary Slitter | Scope: Met Slitter-1 & 2 |
|  | vkstech.com ¦ Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Create joints when transitioning between jumbos or removing defective sections. Joint quality directly affects downstream lamination performance.

👤 Responsibility

Slitting Operator (execution). QC Officer (quality verification).

📋 Procedure

Step 1 - Stop Machine

STOP before jumbo finishes or before defective section. Never let film run out - uncontrolled end causes contamination.

Step 2 - Cut Film

Cut cleanly at joint scale. Cut must be straight across full width - angled cut creates uneven joint.

Step 3 - Position Next Film

Pass next jumbo film to joint scale. Pull tight to eliminate slack.

Step 4 - Apply Joint Tape

DOUBLE-SIDED tape for primary bond. Then SINGLE-SIDED tape on top for reinforcement. This creates a proper butt joint.

Step 5 - Tag Joint Rolls

JOINT TAG on every slit roll containing a joint. Tag must be visible from outside wrapped roll.

Step 6 - Record Details

In logbook: which rolls have joints, position (length from top), source (jumbo transition or defect removal).

📊 Joint Count Limits

| Order Type | Maximum Joints |
| --- | --- |
| Domestic | 2 joints per roll |
| Export | 1 joint per roll |

Poor joint = web break at customer laminator = production line stoppage = angry customer call.