|  | VKS TECH Met Operational Excellence | Doc: VKS/MET_SL/WI/07 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Slit Roll Unloading and Wrapping | Scope: Met Slitter-1 & 2 |
|  | vkstech.com ¦ Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Safe unloading and correct wrapping of finished slit rolls to protect metallized surface during storage and transport.

👤 Responsibility

Slitter Operator (execution). QC Officer (verification before wrapping).

📋 Procedure

Step 1 - Quality Verification

ALL rolls checked by slitting officer AND QC officer. No roll wrapped until QC approves. Rejected rolls kept separately.

Step 2 - Wrapping Sequence (Critical Order)

First layer: Metallized film wrap keeping METAL SIDE OUT. This protects the surface from scratches.

Second layer: BLUE coloured Air Bubble wrap (100 GSM). This provides cushioning for transport.

Step 3 - Conveyor Setup

Conveyor in forward position with Plastic Pallets at each roll position. Number of pallets must EXACTLY match number of slit rolls.

Step 4 - Unload Rolls

Operate rewinder beam from winding to unload position using UNLOAD PUSH BUTTON. Beam moves down until rolls rest on pallets.

Step 5 - Verify Pallets

Visually confirm ALL pallets correctly positioned under rolls. Misaligned pallet = roll drops to floor = surface damage.

Step 6 - Release

Press UNCHUCK BUTTON to release shaft. Rolls now on pallets, ready for weighing station.

Wrapping order METAL-OUT first, then blue bubble is NON-NEGOTIABLE. Reversed wrapping = bubble texture imprints on metallized surface = customer rejection. This mistake has caused rejections worth lakhs.