|  | VKS TECH Met Operational Excellence | Doc: VKS/MET_SL/WI/08 Rev: 01 April 2026 |
| --- | --- | --- |
|  | Data Entry for Roll and Roll ID Nomenclature | Scope: Slitter Area |
|  | vkstech.com ¦ Industrial Knowledge for Packaging Professionals |  |

🎯 Purpose

Correct data entry and Roll ID coding system that enables complete traceability from finished roll back to bare film source.

👤 Responsibility

Slitter Engineer / Data Entry Operator (entry). Supervisor (verification).

📋 Data Entry Procedure

Step 1 - Weigh All Rolls

Every finished roll weighed on weighing conveyor. Record gross (with core) and net (film only) weight.

Step 2 - Enter in SAP

Enter all roll data in SAP system per VKS/MET_SL/WI/10 procedure.

Step 3 - Print and Paste Labels

Print roll labels, paste on each roll surface. Must be visible after wrapping.

📊 Roll Label Contains

Order No., Product Code, Type, Thickness (micron), Width (mm), Length (m), Net Weight (kg), Gross Weight (kg), Treatment Side, Joints, Remarks, Roll ID.

🔤 Roll ID Nomenclature

| Position | Meaning | Values | Example |
| --- | --- | --- | --- |
| 1st | Plant identifier | G, V, etc. | G |
| 2nd | Film Type | M = Metallised | M |
| 3rd | Machine | A=Met-1, B=Met-2 | A |
| 4-5th | Year | 25=2025, 26=2026 | 25 |
| 6th | Month code | A=Jan...L=Dec | F (June) |
| 7-10th | Serial number | 0001-9999 | 0001 |
| 11th | Roll position | L=Left, C=Center, R=Right | C |

Example: GMA25F0001C = Plant G, Metallised, Met-1, 2025, June, Serial 0001, Center.

Roll ID is the ONLY way to trace a customer complaint. Wrong ID = untraceable = audit failure. Double-check every character.