| VKS TECH | INTEGRATED MANAGEMENT SYSTEM vkstech.com | FORMAT NO: VKS/MET/F/11 ISSUE NO: 01 REV: 00 EFFECTIVE: 01.04.2026 PAGE: Page 1 of 2 |
| --- | --- | --- |
|  | Metalliser PM Check List — QUARTERLY & ANNUALLY (Met-2) |  |

| YEAR |  | MACHINE | Met-2 | DEPARTMENT | Metalliser Process |
| --- | --- | --- | --- | --- | --- |

▸ Checklist for "QUARTERLY" Inspection by Process Team

| S.No | असेंबली का नाम (Assembly Name) | करने का कार्य (Task Description) | Date-1 | Date-2 | Date-3 | Date-4 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Cryo-Generator – Brookes Maxcool Mk 2 | रेफ्रिजरेशन सिस्टम बैलेंस प्रेशर चेक करें, HMI इंटरफेस का उपयोग करके डेटा दर्ज करें। |  |  |  |  |
| 2 | Vacuum System Performance | वैक्यूम चेंबर, पंप और पाइपवर्क की लीकेज जांच के लिए हीलियम लीक डिटेक्टर का उपयोग करें। |  |  |  |  |
| 3 | GRE Chiller Unit (48KW-96KW) | इंटरफेस स्क्रीन पर सक्शन और डिस्चार्ज प्रेशर की जांच करें। |  |  |  |  |
| 4 | GRE Chiller Unit (48KW-96KW) | वैक्यूम मोड में फीड और रिटर्न वाटर टेम्परेचर मापें, अंतर रिकॉर्ड करें; कंडेंसर ब्लॉकेज का संकेत मिलेगा। |  |  |  |  |
| 5 | GRE Chiller Unit (48KW-96KW) | पंप और फ्लो स्विच का सही संचालन जांचें और HMI पर दिखाए गए मूल्यों को रिकॉर्ड करें। |  |  |  |  |
| 6 | GRE Chiller Unit (48KW-96KW) | कूलेंट (ग्लाइकोल) की सांद्रता की जांच करें। नमूना हॉट टैंक से लें। |  |  |  |  |
| 7 | GRE Chiller Unit (48KW-96KW) | ग्लाइकोल की गुणवत्ता और प्रदूषण की जांच करें, आवश्यकतानुसार मिश्रण बदलें, फिल्टर और पाइप साफ करें। |  |  |  |  |
| 8 | MKS Baratron Vacuum Gauge | प्लाज़्मा ट्रीटर से फेसप्लेट तक पाइपवर्क की आंतरिक स्थिति की दृश्य जांच करें। |  |  |  |  |
| 9 | Coating Shield & Shutter | शटर सिलेंडर हाउसिंग को खोलें और गाइड रेल, रोलर, सिलेंडर को साफ करें। |  |  |  |  |
| 10 | Aluminium Wire Feeder Units | प्लास्टिक इनफीड बुश की स्थिति की जांच करें। |  |  |  |  |
| 11 | Aluminium Wire Feeder Units | बेयरिंग के घूर्णन की जांच करें। |  |  |  |  |
| Checked By Name: |  |  |  |  |  |  |
| Checked by Signatures: |  |  |  |  |  |  |

| VKS TECH | INTEGRATED MANAGEMENT SYSTEM vkstech.com | FORMAT NO: VKS/MET/F/11 ISSUE NO: 01 REV: 00 EFFECTIVE: 01.04.2026 PAGE: Page 2 of 2 |
| --- | --- | --- |
|  | Oil & Coolant Replacement Record — Met-2 (3650MM / VENTASSO) |  |

| YEAR |  | MACHINE | Met-2 |
| --- | --- | --- | --- |

▸ Oil & Coolant Replacement Record — Met-2 (3650MM / VENTASSO)

| S.No | Equipment | Frequencies | Oil/Coolant Type | Date-1 | Date-2 | Next Due | Date-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Aerzen mechanical booster pumps | Annual, Removal, Strip Down | Anderol 555 |  |  |  |  |
| 2 | Rotary Pump Busch dry pumps (Oil) | Annual, Removal, Strip Down | Anderol 555 |  |  |  |  |
| 3 | Rotary Busch dry pumps (Coolant - 5000 Hrs) | Annual, Strip Down | 40% Glycol + 60% DM Water |  |  |  |  |
| 4 | Diffusion pumps | Annual, Removal, Strip Down | Bobst CVC Silicone 4 |  |  |  |  |
| 5 | Winding Cart Movement Gearbox | Annual, Removal, Strip Down | BP Energol GR-XP 220 |  |  |  |  |
| 6 | Drum leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 10 |  |  |  |  |
| 7 | DTR1 roller leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 10 |  |  |  |  |
| 8 | Unwind chuck gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 9 | Draw | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 10 | Rewind chuck gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 11 | Draw roller leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 12 | DTR2 roller leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 13 | Viewing flap valve leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 14 | Shutter drive leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 15 | High vacuum valve (VV7) leadthrough gravity bottle | Removal, Strip Down | Castrol Hyspin AWS 68 |  |  |  |  |
| 16 | Chiller Compressor | Removal, Strip Down | Polyolester Oil, ICI Emkarate 32 3MAF |  |  |  |  |
| 17 | Brookes Maxcool Unit Compressor | Removal, Strip Down | Solest LT-32 |  |  |  |  |
| 18 | Lauda Refrigeration System | Strip Down | Solest LT-32 |  |  |  |  |
| Checked By Name: |  |  |  |  |  |  |  |
| Checked by Signatures: |  |  |  |  |  |  |  |