| VKS TECH | VKS TECH ¦ INTEGRATED MANAGEMENT SYSTEM vkstech.com VKS POLYFILMS PVT LIMITED | FORMAT NO: VKS/MET/F/12 Issue No: 01 Revision No: 0.00 Effective Date: 01.04.2026 |
| --- | --- | --- |
|  | Metalliser PM Checklist — MONTHLY + QUARTERLY — Mechanical Dept (BOBST K5 Expert) |  |

Monthly: 35 checkpoints  |  Quarterly: 1 checkpoint  |  Tick (✓) = Done  |  Cross (✗) = Not done, write reason in Remarks

▶  MONTHLY INSPECTION  (35 Checkpoints)  —  Perform Once Per Month

| S.NO. | ASSEMBLY NAME | TASK DESCRIPTION | DATE-1 | DATE-2 | DATE-3 | DATE-4 | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | BUSH 630C Screw Pump | Make visual inspection of the pump. If operating in a dusty environment, ensure room is clean and free of dust; clean if necessary. |  |  |  |  |  |
| 2 | BUSH 630C Screw Pump | If discharge is equipped with a silencer, bleed condensation through the purge system. |  |  |  |  |  |
| 3 | BUSH 630C Screw Pump | Check the level of internal cooling liquid (see "Checking the Cooling Liquid" in vendor manual). |  |  |  |  |  |
| 4 | Mechanical Booster Pumps | Check locating and pipework bolts for tightness. |  |  |  |  |  |
| 5 | Mechanical Booster Pumps | Check condition of CM2000 booster pump inlet filters and clean as necessary per the cleaning section of the manual. |  |  |  |  |  |
| 6 | Vacuum Pipework Isolation Valves | Grease rotary leadthroughs with Mobil Supertemp SHC32 grease. |  |  |  |  |  |
| 7 | Vacuum Pipework Isolation Valves | Check location bolts/nuts are tight. |  |  |  |  |  |
| 8 | Vacuum Pipework Isolation Valves | Check external ETP bush has not moved (witness marks provided); check cylinder locking nut is tight; check rod "fork" is correctly attached. |  |  |  |  |  |
| 9 | Chamber Vent Valves | Check condition of any filters installed — clean or replace as required. Monitor time for chamber to vent to atmosphere: this indicates replacement timing for internal filter media. |  |  |  |  |  |
| 10 | Chamber Vent Valves | Clean the internal deflection plate inside the chamber. |  |  |  |  |  |
| 11 | Vacuum Pipeline Rubber Bellows | Check tightness of hose clamps — two on either side of joint, clamp sections 180° apart to prevent bellows being made oval. |  |  |  |  |  |
| 12 | GRE Chiller Unit (48KW–96KW) | Check that external panel location fasteners are fully engaged. |  |  |  |  |  |
| 13 | Evaporation Source Viewing Flap | Check universal joint and splines for correct movement. |  |  |  |  |  |
| 14 | Coating Shield & Shutter | Apply Grease Type A (Mobil SHC32 or equivalent) to pivot bearing housings and winch shaft bearing. |  |  |  |  |  |
| 15 | Aluminium Wire Feeder Units | Check for correct compression of spring and ease of movement of pivot. |  |  |  |  |  |
| 16 | Evaporation Source Area | Check tightness of copper block retaining bolts. |  |  |  |  |  |
| 17 | Substrate Winding Mechanism | Visually inspect upper castors (front and back) and internal chamber rails for indication of wear. |  |  |  |  |  |
| 18 | Chucking System & Reel Shafts | Check two reel shaft external bearings for wear. Grease lightly with correct grease. Check circlips are fully engaged. |  |  |  |  |  |
| 19 | Chucking System & Reel Shafts | Check contact areas on reel shaft for damage — hardened surfaces. Smooth any high spots. |  |  |  |  |  |
| 20 | Chucking System (Shaftless – Pneumatic) | Grease the inboard and outboard chuck linear bearings. |  |  |  |  |  |
| 21 | Winding Mechanism Belt Drive System | Check external drive belts and pulleys for wear and correct alignment. Check belt tensions per manufacturer method in vendor manual. |  |  |  |  |  |
| 22 | Winding Cart Traverse Drive Motor & Gearbox | Check motor/gearbox for physical damage. |  |  |  |  |  |
| 23 | Winding Cart Traverse Drive Motor & Gearbox | Visually check chain drive for wear and tightness. Lubricate with industry-standard light chain oil. |  |  |  |  |  |
| 24 | Winding Cart Traverse Drive Motor & Gearbox | Check brass locating block for wear (excess wear indicates cart misalignment); check locating bolts are tight and locking nuts are engaged. |  |  |  |  |  |
| 25 | Winding Cart Traverse Drive Motor & Gearbox | Check condition of cart castors; check for excess wear on tracks. Castor alignment may be required. |  |  |  |  |  |
| 26 | Ext. Cooled Rollers Rotary Union (Deublin) | Lubricate the union and check for leaks. |  |  |  |  |  |
| 27 | Electrical Panels & AC Units | Wipe down heat exchanger fins after removing access panels. |  |  |  |  |  |
| 28 | Planar Plasma Treater | Check tightness of all fixing components. |  |  |  |  |  |
| 29 | Machine Water Services | Open drain valve on main filter unit to bleed off any entrapped debris. |  |  |  |  |  |
| 30 | Machine Water Services – Main Line Strainer | Remove, clean and reinstall the main water line strainer mesh. Check mesh for damage; replace if torn or deformed. Record condition. |  |  |  |  |  |
| 31 | Advanced Energy Plasma Power Supply | Check that water cooling lines to power supply are not damaged and show no indication of condensation. |  |  |  |  |  |
| 32 | Advanced Energy Plasma Power Supply | Clean/replace filter material in the rear panel door. |  |  |  |  |  |
| 33 | Hawkeye Inline Optical & Defect Monitor | Check mounting assembly is correctly located; check beam location/jacking screws are not loose (do not overtighten — will move system alignment). |  |  |  |  |  |
| 34 | Winding Mechanism – Web Threader | Check that all pulleys are free to rotate. |  |  |  |  |  |
| 35 | Gearboxes | Check all gearboxes for indication of leakage and resolve any found. Change oil or grease where possible per manufacturer’s manual. |  |  |  |  |  |
| Checked By Name: |  |  |  |  |  |  |  |
| Checked By Signature: |  |  |  |  |  |  |  |

▶  QUARTERLY INSPECTION  (1 Checkpoint)  —  Perform Once Every 3 Months

| S.NO. | ASSEMBLY NAME | TASK DESCRIPTION | DATE-1 | DATE-2 | DATE-3 | DATE-4 | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GRE Chiller Unit – Water Circuit Strainer | Remove and clean the GRE chiller line strainer mesh on all chiller water circuit connections. Inspect mesh for damage, corrosion or fouling; replace if required. Flush strainer housing before reinstalling. Record condition and date. |  |  |  |  |  |
| Checked By Name: |  |  |  |  |  |  |  |
| Checked By Signature: |  |  |  |  |  |  |  |