| VKS TECH | VKS TECH ¦ INTEGRATED MANAGEMENT SYSTEM vkstech.com VKS POLYFILMS PVT LIMITED | FORMAT NO: VKS/MET/F/17 Issue No: 01 Revision No: 0.00 Effective Date: 01.04.2026 |
| --- | --- | --- |
|  | Metalliser PM Checklist — WEEKLY — Mechanical Dept (BOBST K5 Expert) |  |

Weekly Checkpoints: 36  |  Tick (✓) = Done  |  Cross (✗) = Not done, write reason in Remarks  |  N/A = Not applicable

▶  WEEKLY INSPECTION  (36 Checkpoints)  —  Perform Every Week

| S.NO. | ASSEMBLY NAME | TASK DESCRIPTION | DATE-1 | DATE-2 | DATE-3 | DATE-4 | REMARKS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | BUSH 630C Screw Pump | Check pump temperature <65°C and check temperature of cooling fluid (see vendor manual). If running warm, check water flow to heat exchanger. |  |  |  |  |  |
| 2 | BUSH 630C Screw Pump | Check pump for no indication of leakage of internal coolant through the pressure relief valve. |  |  |  |  |  |
| 3 | BUSH 630C Screw Pump | Check pump area for oil leaks and water leaks. Repair as necessary. |  |  |  |  |  |
| 4 | BUSH 630C Screw Pump | Visually check pump oil level per sheet attached to pump or refer to vendor manual. |  |  |  |  |  |
| 5 | BUSH 630C Screw Pump | Check air is being fed through dilution port (place hand near filter to feel air draw). Ensure filter is clean; replace if necessary. |  |  |  |  |  |
| 6 | Vacuum Pipeline Rubber Bellows | Visually check bellows for wear or damage. Replace as necessary. |  |  |  |  |  |
| 7 | Vacuum Pipeline Rubber Bellows | Check for excess movement — this can lead to air leaks. |  |  |  |  |  |
| 8 | Vacuum Chamber | Clean and lubricate winding mechanism joint face O-ring with suitable grease. |  |  |  |  |  |
| 9 | Evaporation Source Viewing Flap | Grease bearings and rotary pivot points at either end of the assembly. |  |  |  |  |  |
| 10 | Evaporation Source Viewing Flap | Visually check that coupling between rotary strobe and motor is correctly engaged with a 3mm space. |  |  |  |  |  |
| 11 | Evaporation Source Viewing Flap | Check that there is no binding between the components. |  |  |  |  |  |
| 12 | Evaporation Source Viewing Flap | Visually check rotary vacuum leadthrough externally for indication of oil loss. Investigate if found. |  |  |  |  |  |
| 13 | Coating Shield & Shutter | Grease the shutter movement mechanism bearings. |  |  |  |  |  |
| 14 | Coating Shield & Shutter | Check shutter for mechanical damage; visually check water supply hoses for indication of cracking or physical wear. |  |  |  |  |  |
| 15 | Substrate Winding Mechanism | Check all carbon fibre non-driven idler rollers by hand for bearing wear and stiffness. Check surface for chip and physical damage; repair with suitable repair gel. |  |  |  |  |  |
| 16 | Substrate Winding Mechanism | Check layarm assembly positioning belts for tooth/belt wear. Check drive pulleys are fully engaged and retaining sides are in place. |  |  |  |  |  |
| 17 | Substrate Winding Mechanism | Grease the layarm linear bearings on unwind and rewind assemblies. |  |  |  |  |  |
| 18 | Substrate Winding Mechanism | Grease unwind spreader roller and unwind layarm roller at both ends. |  |  |  |  |  |
| 19 | Substrate Winding Mechanism | Grease load cell roller transducer bearings at both roller ends. |  |  |  |  |  |
| 20 | Substrate Winding Mechanism | Grease non-driven end bearings at faceplate: process drum, DTR2, DTR1, and draw roller. |  |  |  |  |  |
| 21 | Chucking System & Reel Shafts | Check chuck support surfaces and rotating chucks for wear and security. |  |  |  |  |  |
| 22 | Chucking System & Reel Shafts | Check swing-over clamps are free to move; check clamp requires pressure to fully close (spring operational); check toggle clamp is not damaged and applies pressure when engaged. |  |  |  |  |  |
| 23 | Chucking System & Reel Shafts | Check reel bar does not "rock" when moved side to side in chuck — indicates worn wear plates requiring replacement. |  |  |  |  |  |
| 24 | Chucking System & Reel Shafts | Check manual clamps engage over the outside of the chuck without excess force. |  |  |  |  |  |
| 25 | Chucking System (Shaftless – Pneumatic) | Check drive surfaces and expanding engagement segments for damage and wear. (These are wear parts — replacement timing depends on cores used.) |  |  |  |  |  |
| 26 | Chucking System (Shaftless – Pneumatic) | Check hydraulic lines and cylinder for indication of oil leaks. Repair as necessary. |  |  |  |  |  |
| 27 | Chucking System (Shaftless – Pneumatic) | Grease the core support assembly. |  |  |  |  |  |
| 28 | Chucking System (Shaftless – Pneumatic) | Check that the core movement limiter is stopping the core from sliding over it. |  |  |  |  |  |
| 29 | Cart Cable Coiler | Check all coiler segments are in place; check entrance/exit to ensure cables/hoses are not being damaged by coiler movement. |  |  |  |  |  |
| 30 | Drum Coating Shield / Zone Sealing | Check condition of zone sealing seals. Renew flexible sections as required. |  |  |  |  |  |
| 31 | Ext. Cooled Rollers Rotary Union (Deublin) | Visually check area around union for indication of liquid (glycol) leakage; remove any excess grease pushed out of union. |  |  |  |  |  |
| 32 | Evaporation Source Electrical Busbar | Visually check for indication of overheating of internal water lines around the busbar system. |  |  |  |  |  |
| 33 | Winding System Drive Belt Guards | Check all guards are in position and locating fasteners are in place; check access door to winding cart lock is operational. |  |  |  |  |  |
| 34 | Bowed Spreader Roller (Kickert) | Rotate roller by hand and check for uneven rotation. If uneven, roller may require overhauling. |  |  |  |  |  |
| 35 | Winding Mechanism – Web Threader | Check condition of belt (replace if worn); check tension is not excessive; check for and remove any build-up of tape on belt. |  |  |  |  |  |
| 36 | Gearboxes | Check all gearboxes for excessive noise where possible. |  |  |  |  |  |
| Checked By Name: |  |  |  |  |  |  |  |
| Checked By Signature: |  |  |  |  |  |  |  |